Федеральное государственное автономное образовательное

учреждение высшего образования

Университет ИТМО

Дисциплина: Администрирование систем и сетей Лабораторная работа 3

Вариант ???

Работу выполнили студенты:

Трикашный М.Д P3406

Кравцев В.В P3414

Желаемая оценка: 3

Преподаватель:

Афанасьев Дмитрий Борисович

2026 г.

Санкт-Петербург

Основы Ethernet и конфигурирование VLAN

Топология сети

Конфигурация

Отключение ненужных портов GE0/0/11 и GE0/0/12 на S1 и на S2

[S1]interface g0/0/11

[S1-GigabitEthernet0/0/11]shutdown

[S1]interface g0/0/12

[S1-GigabitEthernet0/0/12]shutdown

[S2]interface g0/0/11

[S2-GigabitEthernet0/0/11]shutdown

[S2]interface g0/0/12

[S2-GigabitEthernet0/0/12]shutdown

Настройка IP-адресов устройств

Установка IP-адресов для R1 и R3.

[R1]interface g0/0/1

[R1-GigabitEthernet0/0/1]ip address 10.1.2.1 24

[R3]interface g0/0/2

[R3-GigabitEthernet0/0/2]ip address 10.1.10.1 24

Установка IP-адресов для VLANIF3 на S3 и S4. Создание VLAN 3.

[S3]vlan 3

[S4]vlan 3

Настройка портов на S3 и S4 в качестве портов доступа и назначение их в соответствующие VLAN.

[S3]interface g0/0/1

[S3-GigabitEthernet0/0/1]port link-type access

[S3-GigabitEthernet0/0/1]port default vlan 3

[S4]interface g0/0/2

[S4-GigabitEthernet0/0/2]port link-type access

[S4-GigabitEthernet0/0/2]port default vlan 3

Создание интерфейсов VLANIF и настройка IP-адреса.

[S3]interface Vlanif 3

[S3-Vlanif3]ip address 10.1.3.1 24

[S4]interface Vlanif 3

[S4-Vlanif3]ip address 10.1.3.2 24

Создание VLAN

Создание VLAN 2, 3 и 10 на S1 и S2.

[S1]vlan batch 2 to 3 10

[S2]vlan batch 2 to 3 10

Настройка сети VLAN на основе портов

Настройка пользовательских портов на S1 и S2 в качестве портов доступа и назначение их в соответствующие VLAN.

[S1]interface g0/0/1

[S1-GigabitEthernet0/0/1]port link-type access

[S1-GigabitEthernet0/0/1]port default vlan 2

[S1]interface g0/0/13

[S1-GigabitEthernet0/0/13]port link-type access

[S1-GigabitEthernet0/0/13]port default vlan 3

[S2]interface g0/0/14

[S2-GigabitEthernet0/0/14]port link-type access

[S2-GigabitEthernet0/0/14]port default vlan 3

Настройка портов, соединяющих S1 и S2, в качестве магистральных портов и разрешение прохождения только пакетов из VLAN 2 и VLAN 3.

[S1]interface g0/0/10

[S1-GigabitEthernet0/0/10]port link-type trunk

[S1-GigabitEthernet0/0/10]port trunk allow-pass vlan 2 3

[S1-GigabitEthernet0/0/10]undo port trunk allow-pass vlan 1

[S2]interface g0/0/10

[S2-GigabitEthernet0/0/10]port link-type trunk

[S2-GigabitEthernet0/0/10]port trunk allow-pass vlan 2 3

[S2-GigabitEthernet0/0/10]undo port trunk allow-pass vlan 1

Сконфигурируйте сети VLAN на основе MAC-адресов

Настройка на S2 привязки MAC-адреса ПК к VLAN 10.

[S2]vlan 10

[S2-vlan10]mac-vlan mac-address a008-6fe1-9c46

Настройка GigabitEthernet0/0/1, GigabitEthernet0/0/2 и GigabitEthernet0/0/3 на S2 в качестве гибридных портов и разрешение прохождения пакетов из VLAN на основе MAC-адресов.

[S2-vlan10]interface g0/0/1

[S2-GigabitEthernet0/0/1]port link-type hybrid

[S2-GigabitEthernet0/0/1]port hybrid untagged vlan 10

[S2]interface g0/0/2

[S2-GigabitEthernet0/0/2]port link-type hybrid

[S2-GigabitEthernet0/0/2]port hybrid untagged vlan 10

[S2]interface g0/0/3

[S2-GigabitEthernet0/0/3]port link-type hybrid

[S2-GigabitEthernet0/0/3]port hybrid untagged vlan 10

Настройка на портах, соединяющих S1 и S2, разрешения на прохождение пакетов из VLAN 10.

[S1]interface g0/0/10

[S1-GigabitEthernet0/0/10]port trunk allow-pass vlan 10

[S2]interface g0/0/10

[S2-GigabitEthernet0/0/10]port trunk allow-pass vlan 10

Настройка S2 и включение назначения VLAN на основе MAC-адресов на GE0/0/1, GE0/0/2 и GE0/0/3.

[S2]interface g0/0/1

[S2-GigabitEthernet0/0/1]mac-vlan enable

[S2]interface g0/0/2

[S2-GigabitEthernet0/0/2]mac-vlan enable

[S2]interface g0/0/3

[S2-GigabitEthernet0/0/3]mac-vlan enable

Вывод информации о конфигурации

[S1]dis vlan The total number of vlans is : 4 -------------------------------------------------------------------------------- U: Up; D: Down; TG: Tagged; UT: Untagged; MP: Vlan-mapping; ST: Vlan-stacking; #: ProtocolTransparent-vlan; *: Management-vlan; -------------------------------------------------------------------------------- VID Type Ports -------------------------------------------------------------------------------- - common UT:GE0/0/2(D) GE0/0/3(D) GE0/0/4(D) GE0/0/5(D) GE0/0/6(D) GE0/0/7(D) GE0/0/8(D) GE0/0/9(D) GE0/0/11(D) GE0/0/12(D) GE0/0/14(D) GE0/0/15(D) GE0/0/16(D) GE0/0/17(D) GE0/0/18(D) GE0/0/19(D) GE0/0/20(D) GE0/0/21(D) GE0/0/22(D) GE0/0/23(D) GE0/0/24(D) - common UT:GE0/0/1(U) TG:GE0/0/10(U) - common UT:GE0/0/13(U) TG:GE0/0/10(U) 10 common TG:GE0/0/10(U) VID Status Property MAC-LRN Statistics Description -------------------------------------------------------------------------------- - enable default enable disable VLAN 0001 - enable default enable disable VLAN 0002 - enable default enable disable VLAN 0003 10 enable default enable disable VLAN 0010

[S2]dis vlan The total number of vlans is : 4 -------------------------------------------------------------------------------- U: Up; D: Down; TG: Tagged; UT: Untagged; MP: Vlan-mapping; ST: Vlan-stacking; #: ProtocolTransparent-vlan; *: Management-vlan; -------------------------------------------------------------------------------- VID Type Ports -------------------------------------------------------------------------------- 1 common UT:GE0/0/1(U) GE0/0/2(D) GE0/0/3(D) GE0/0/4(D) GE0/0/5(D) GE0/0/6(D) GE0/0/7(D) GE0/0/8(D) GE0/0/9(D) GE0/0/11(D) GE0/0/12(D) GE0/0/13(D) GE0/0/15(D) GE0/0/16(D) GE0/0/17(D) GE0/0/18(D)

GE0/0/19(D) GE0/0/20(D) GE0/0/21(D) GE0/0/22(D) GE0/0/23(D) GE0/0/24(D) - common TG:GE0/0/10(U) - common UT:GE0/0/14(U) TG:GE0/0/10(U) 10 common UT:GE0/0/1(U) GE0/0/2(D) GE0/0/3(D) TG:GE0/0/10(U) VID Status Property MAC-LRN Statistics Description -------------------------------------------------------------------------------- - enable default enable disable VLAN 0001 - enable default enable disable VLAN 0002 - enable default enable disable VLAN 0003 10 enable default enable disable VLAN 0010

[S2]dis mac-vlan vlan 10

---------------------------------------------------

MAC Address     MASK            VLAN    Priority

--------------------------------------------------a008-6fe1-9c46  ffff-ffff-ffff  10

0 Total MAC VLAN address count: 1

Проверка

Ping на S4 для проверки связи с S3. Операция успешно выполняется:

[S4]ping 10.1.3.2 PING 10.1.3.2: 56 data bytes, press CTRL_C to break Reply from 10.1.3.2: bytes=56 Sequence=1 ttl=255 time=30 ms Reply from 10.1.3.2: bytes=56 Sequence=2 ttl=255 time=30 ms Reply from 10.1.3.2: bytes=56 Sequence=3 ttl=255 time=30 ms Reply from 10.1.3.2: bytes=56 Sequence=4 ttl=255 time=1 ms Reply from 10.1.3.2: bytes=56 Sequence=5 ttl=255 time=20 ms --- 10.1.3.2 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 1/22/30 ms

Ping на R1 для проверки связи с другими устройствами. Операция не выполняется:

[R1]ping 10.1.3.1 PING 10.1.3.1: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.1.3.1 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [R1]ping 10.1.3.2 PING 10.1.3.2: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.1.3.2 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [R1]ping 10.1.10.1 PING 10.1.10.1: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 10.1.10.1 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss

Таблицы MAC-адресов на коммутаторах

[S2]dis mac-address verbose MAC address table of slot 0: ------------------------------------------------------------------------------- MAC Address VLAN/ PEVLAN CEVLAN Port Type LSP/LSR-ID VSI/SI MAC-Tunnel ------------------------------------------------------------------------------- 4c1f-cc91-281d 3 - - GE0/0/14 dynamic 0/- ------------------------------------------------------------------------------- Total matching items on slot 0 displayed = 1

Вывод

Мы снова делали ctrl-c ctrl-v из методички и топология заработала!