Федеральное государственное автономное образовательное

учреждение высшего образования

Университет ИТМО

Дисциплина: Администрирование систем и сетей Лабораторная работа 4

Вариант ???

Работу выполнили студенты:

Трикашный М.Д. P3406

Кравцев В.В. P3414

Желаемая оценка: 3

Преподаватель:

Афанасьев Дмитрий Борисович

2026 г.

Санкт-Петербург

Настройка NAT

Топология

Настройка основных параметров

#Настройка IP-адресов и маршрутов

[AR1]int g0/0/0

[AR1-GigabitEthernet0/0/0]ip ad 192.168.1.1 24

[AR1]ip route-static 0.0.0.0 0 192.168.1.254

[AR2]int g0/0/0

[AR2-GigabitEthernet0/0/0]ip ad 192.168.1.254 24

[AR2-GigabitEthernet0/0/0]int g0/0/1

[AR2-GigabitEthernet0/0/1]ip ad 1.2.3.4 24

[AR2]ip route-static 0.0.0.0 0 1.2.3.254

[AR3]int g0/0/0

[AR3-GigabitEthernet0/0/0]ip ad 1.2.3.254 24

#Настройка функции telnet на AR1 и AR3

[AR1]user-int vty 0 4

[AR1-ui-vty0-4]authentication-mode aaa

[AR1-ui-vty0-4]q

[AR1]aaa

[AR1-aaa]local-user test password cipher Huawei@123

Info: Add a new user.

[AR1-aaa]local-user test service-type telnet

[AR1-aaa]local-user test privilege level 15

[AR3]user-interface vty 0 4

[AR3-ui-vty0-4]authentication-mode aaa

[AR3-ui-vty0-4]q

[AR3]aaa

[AR3-aaa]local-user test password cipher Huawei@123

Info: Add a new user.

[AR3-aaa]local-user test service-type telnet 
[AR3-aaa]local-user test privilege level 15

Проверка доступа

[AR1]ping 1.2.3.254 PING 1.2.3.254: 56 data bytes, press CTRL_C to break Request time out Request time out Request time out Request time out Request time out --- 1.2.3.254 ping statistics --- 5 packet(s) transmitted 0 packet(s) received 100.00% packet loss [AR2]ping 1.2.3.254 PING 1.2.3.254: 56 data bytes, press CTRL_C to break Reply from 1.2.3.254: bytes=56 Sequence=1 ttl=255 time=50 ms Reply from 1.2.3.254: bytes=56 Sequence=2 ttl=255 time=20 ms Reply from 1.2.3.254: bytes=56 Sequence=3 ttl=255 time=10 ms Reply from 1.2.3.254: bytes=56 Sequence=4 ttl=255 time=20 ms Reply from 1.2.3.254: bytes=56 Sequence=5 ttl=255 time=20 ms --- 1.2.3.254 ping statistics --- 5 packet(s) transmitted 5 packet(s) received 0.00% packet loss round-trip min/avg/max = 10/24/50 ms

Настройка функции динамического NAT

#Настройка пула адресов NAT

[AR2]nat address-group 1 1.2.3.10 1.2.3.20

#Настройка ACL

[AR2]acl 2000

[AR2-acl-basic-2000]rule 5 permit source any

#Настройка динамического NAT на g/0/0/1 маршрутизатора AR2

[AR2]int g0/0/1

[AR2-GigabitEthernet0/0/1]nat outbound 2000 address-group 1

#Выполнение входа с AR1 на AR3 через telnet для моделирования трафика tcp <AR1>telnet 1.2.3.254 Press CTRL_] to quit telnet mode Trying 1.2.3.254 ... Connected to 1.2.3.254 ... Login authentication Username:test Password: <AR3>

#Вывод на экран таблицы сеансов NAT на AR2 [AR2-GigabitEthernet0/0/1]dis nat session all NAT Session Table Information: Protocol : TCP(6) SrcAddr Port Vpn : 192.168.1.1 20934 DestAddr Port Vpn : 1.2.3.254 5888 NAT-Info New SrcAddr : 1.2.3.16 New SrcPort : 10240 New DestAddr : ---- New DestPort : ---- Total : 1

Настройка Easy IP

#Удаление конфигурации, созданной на предыдущем шаге

[AR2]int g0/0/1

[AR2-GigabitEthernet0/0/1]undo nat outbound 2000 address-group 1

#Настройка easy-ip

[AR2-GigabitEthernet0/0/1]nat outbound 2000

#Выполнение входа с AR1 на AR3 через telnet для моделирования трафика tcp <AR1>telnet 1.2.3.254 Press CTRL_] to quit telnet mode Trying 1.2.3.254 ... Connected to 1.2.3.254 ... Login authentication Username:test Password: <AR3>

#Вывод на экран таблицы сеансов easy-ip на AR2 <AR2>dis nat session all NAT Session Table Information: Protocol : TCP(6) SrcAddr Port Vpn : 192.168.1.1 48836 DestAddr Port Vpn : 1.2.3.254 5888 NAT-Info New SrcAddr : 1.2.3.4 New SrcPort : 10297 New DestAddr : ---- New DestPort : ---- Protocol : TCP(6) SrcAddr Port Vpn : 192.168.1.1 25540 DestAddr Port Vpn : 1.2.3.254 5888 NAT-Info New SrcAddr : 1.2.3.4 New SrcPort : 10298 New DestAddr : ---- New DestPort : ---- Total : 2

Настройка сервера NAT на исходящем интерфейсе AR2

#Настройка сервера NAT на AR2

[AR2]int g0/0/1

[AR2-GigabitEthernet0/0/1]nat server protocol tcp global current-interface 2323 inside 192.168.1.1 telnet

#Выполнение входа с AR3 на AR1 через telnet

<AR3>telnet 1.2.3.4 2323 Press CTRL_] to quit telnet mode Trying 1.2.3.4 ... Connected to 1.2.3.4 ... Login authentication Username:test Password: <AR1>

#Вывод на экран таблицы сеансов NAT на AR2 [AR2-GigabitEthernet0/0/1]dis nat session all NAT Session Table Information: Protocol : TCP(6) SrcAddr Port Vpn : 192.168.1.1 48836 DestAddr Port Vpn : 1.2.3.254 5888 NAT-Info New SrcAddr : 1.2.3.4 New SrcPort : 10297 New DestAddr : ---- New DestPort : ---- Protocol : TCP(6) SrcAddr Port Vpn : 192.168.1.1 25540 DestAddr Port Vpn : 1.2.3.254 5888 NAT-Info New SrcAddr : 1.2.3.4 New SrcPort : 10298 New DestAddr : ---- New DestPort : ---- Protocol : TCP(6) SrcAddr Port Vpn : 1.2.3.254 62402 DestAddr Port Vpn : 1.2.3.4 4873 NAT-Info New SrcAddr : ---- New SrcPort : ---- New DestAddr : 192.168.1.1 New DestPort : 5888 Total : 3

Вывод

В ходе лабораторной работы познакомились с NAT.